Changing IP addresses on iSeries Printers

| Command to see list of IP addressable printers | wrkdevd *lanprt |
| --- | --- |
| Option 8 by the printer to see the status |  |
| 8 |  |
| If writer is started, end writer | Endwtr (writername) *immed |
| Then vary off the printer with option 2 |  |
| Status will go to varied off |  |
| F12 to go back to description |  |
| Option 2 to change |  |
| F10 and page down until see the IP address – change it: |  |
| Press enter. |  |
| Message at bottom of screen |  |
| Option 8 by printer again to vary on. |  |
| Vary on. |  |
| Start printer writer. |  |
| Command: | Strprtwtr prtapprx |
| Change the IP address physically on the printer to match what you changed it to on the AS400/iSeries. |  |
| Test by sending a report to the printer |  |
| Command: | Wrksplf |
| Option 2 by the spooled file and on the command line type outq(nameofprinter) |  |
| Any questions, contact supportdesk@cookconsulting.net |  |